Week1

- Arduino is an open-source computer hardware/software platform for building digital devices and interactive objects that can sense and control the physical world around them

- Arduino boards are able to read inputs - light on sensor, a finger on button, or a twitter message - and turn to it into output - activating a motor, turning an LED light, publishing something online

- The least expensive version of the Arduino module can be assembled by hand

- IoT - internet of things

Arduino Board

- Digital I/O: draws and output 0 to 5 volts

- Analog inputs: draws a volt between 0 and 5

- Power/reset pins: have 5v, 3.3V, ground, so you can use those pins with the outside world

- You code is written only to the ATmega328, the ATmega16U2 talks to the USB and has pre written code which you don't touch it

- A bootloader is an example of Firmware, the code you write is the application

- Update the bootloader

  - To program the circuit with a new bootloader you do not use the normal USB programming. That is done with In-Circuit Serial Programming (ICSP)

https://www.electronicwings.com/arduino/basics-to-developing-bootloader-for-arduino

Arduino Schematics

IDE

Arduino Shields

Week2

Setting up your environment

The benefit of an IDE is that it puts all the tools together in a nice graphical user interface

$ serup() function

The setup() function is called when a sketch starts. Use it to initialize variables, pin modes, start using libraries, etc. the setup() function will only run once, after each powerup or reset of the Arduino board.

Example Code

$ int buttonPin = 3;

$ void setup() {

$        Serial.begin(9600);

$        pinMode(buttonPin, INPUT);

$ }

$ void loop() {

$        // ….

$ }

$ pinMode()

[Digital I/O]

Configure the specfied pin to behave either as an input or an output.

Syntax

$ pinMode(pin, mode)

Prameters

$ pin: the Arduino pin number to set the mode of

$ mode: INPUT, OUTPUT, OR INPUT_PULLUP.

Returns

Nothing

$ loop()

Description

After creating the setup() function, which initializes and sets the initial values, the loop() function does precisely what its name suggests, and loops consecutively, allowing your program to change and respond. Use it to actively control the Arduino board.

$ digitalWrite()

Description

Write a HIGH or a LOW value to a digital pin

Syntax

digitalWrite(pin, value)

Parameters

pin: the Arduino pin number.

value: HIGH or LOW.

Returns

Nothing

Example Code

The code makes the digital pin 13 an OUTPUT and toggles it by alternating between HIGH and LOW at one second pace.

void setup() {
  pinMode(13, OUTPUT);    // sets the digital pin 13 as output
}

void loop() {
  digitalWrite(13, HIGH); // sets the digital pin 13 on
  delay(1000);            // waits for a second
  digitalWrite(13, LOW);  // sets the digital pin 13 off
  delay(1000);            // waits for a second
}

$ digitalRead()

Description

Reads the value from a specified digital pin, wither HIGH or LOW

Syntax

$ digitalRead(pin)

Parameters

$ pin: the Arduino pin number you want to read

Returns

HIGH or LOW

Arduino Toolchain

Cross Compilation

Arduino Programs

- A program is called a sketch

- C++ program using Arduino library functions

- C++ is a superset of C

Classes

Classes basically bring data and functions together and give them name

Week3

Sketch

$ setup() function

- A sketch does not have a main() function

- Every sketch (Arduino program) has a setup() function

  - Executed once when Arduino is powered up

  - Used for initialization operation

  - Returns no value, takes no arguments

  - Void setup()

  - {

  - ….

  - }

$ loop() function

- Every sketch (Arduino program) has a loop() function

  - Executed iteratively as long as the Arduino is powered up

  - loop() starts executing after the setup() has finished

  - loop() is the main() program control flow

  - Return no value, takes no arguments

  - void loop()

  - {

  - ….

  - }

Pins

- Pins are wires connected to the microcontroller

- Pins are the interface of the microcontroller

- Pins voltages are controlled by a sketch

- Pin voltages can be read by a sketch

Pins can be designated as INPUT or OUTPUT

Output pins

Output pins are controlled by the Arduino

Voltage is determined by your sketch

Other components can be controlled through outputs

Input pins

Input pin are controlled by other components

Arduino reads the voltage on the pins

Allows it to respond to events and data

Week4

Debugging

Debug and Trace

Controllability and observability are required

Controllability

- Ability to control sources of data used by the system

- Input pins, input interfaces (serial, ethernet, etc.)

- Registers and internal memory

Observability

- Ability to observe intermediate and final results

- Output pins, output interfaces

- Registers and internal memory

- Oscilloscope (voltage over time)

Properties of a Debugging Environment

- Run control of the target

- Start and stop the program execution

- Observe data at stop points

- Real-time monitoring of target execution

- Non-intrusive in terms of performance

- Timing and functional accuracy

- Debugged system should act like real system

Debug

Remote Debugger

'

Debug via Serial

Serial Protocols

- Data is transmitted serially

  - Only 1 bit needed (plus common ground)

- Parallel data transmitted serially

- Original bytes/words regrouped by receiver

- Many protocols are serial to reduce pin usage

  - Pins are precious

UART

- Universal Asynchronous receiver/Transmitter

- Used for serial communication between devices

- UART is asynchronous no shared clock (no clock), hence it work over lock distance

- Clock is a square wave

- Asynchronous allows longer distance communication

  - Clock skew is not a problem

UART Applications

- Used by modems to communicate with network

- Computers used to have an RS232 port standard

- Not well used anymore, outside of embedded systems

  - Replaced by USB, ethernet, 12C, SPI

- Simple , low HW overhead

- Built into most microcontrollers

UART Protocol

- Data is serialized by Tx, deserialized by Rx

- Status indicates the state of the transmit/receive buffers

  - Useful for flow control

- First bit is the start bit; indicates the transfer

- Next bits are the data

- Last are two bits are the Stop Bits

Bit duration

- Each bit is transmitted for fixed duration

- The duration must be known to Tx and Rx

- Baud rate (f) determines the duration (T)

- Baud rate is the number of transitions per second

  - Typically measured in "bits per second (bps)"

- T = 1/f

  - F = 9600 baud, T = ~104 microsecond

- Transmission rate is less than baud rate

UART Synchronization

The receiver should know when data send

UART Parity and Stop